Inverter terminal descriptions

### Primary Circuitry Terminal Block (TM1) descriptions

| ### Terminal Symbol | ### Function Description |
| --- | --- |
| L1/L (R) | Primary power source input to Drive Single phase: L1/L2 or L/N Three phase: L1/L2/L3 |
| L2 (S) |  |
| L3/N (T) |  |
| P | Extermal braking resistor terminal (Only for E2-202/203/401/402/403) |
| R |  |
| T1 (U) | Inverter output to Motor |
| T2 (V) |  |
| T3 (W) |  |

Tightening torque for TM1 is 1 LBS-FT or 12 LBS-IN (1P2/1P5/2P2/2P5/201).

Tightening torque for TM1 is 1.3 LBS-FT or 16 LBS-IN (202/203/401/402/403).

* Wire voltage rating must be a minimum of 300V(200V series)/600V(400V series)

### Control Circuitry Terminal Block (TM2) description

| ### Terminal Symbol |  | ### Function Description |  |
| --- | --- | --- | --- |
| 1 | TRIP RELAY | Fault relay output terminal ﹠Multi function output terminal (refer to F_21) Connection point rated capacity 250VAC/1A (30VDC / 1A) |  |
| 2 |  |  |  |
| 3 | FWD (FW) | Operation control terminals (refer to F_03) |  |
| 4 | REV (RE) |  |  |
| 5 | + 12V(12) | Common point of terminal 3 / 4 / 6 / 7 |  |
| 6 | SP1(SP) | Multifunction input terminals (refer to F_19) |  |
| 7 | RESET(RS) |  |  |
| 8 | 0V(FM -) | +10V | Power terminal for potentiometer ( Pin 3 ) |
| 9 |  | Analog input wire Wiper | Analog frequency signal input terminal ( Pin 2 of potentiometer or positive terminal of 0~10V / 4~20mA / 0~20mA) |
| 10 |  | Analog common point | Analog signal common point ( Pin 1 of potentiometer or negative terminal of 0~10V / 4~20mA / 0~20mA ) |
| 11 | FM+ | Analog output positive connection point | Analog frequency signal output terminal Output terminal signal is 0 ~ 10VDC/Fn6 |

Tightening torque for TM2 is 0.42 LBS-FT or 5.03 LBS-IN.

* Wire voltage rating must be a minimum of 300V

* Control wiring should not run in the same conduit or raceway with power or motor wiring

* Single Input and Output Terminals (TM2) Ratings are ALL Class 2

SW1 function description

| SWITCH 1 | External signal type |
| --- | --- |
|  | 0~20mA analog signal (When F_11 is set to 1) 4~20mA analog signal (When F_11 is set to 2) |
|  | 0~10 VDC analog signal (When F_11 is set to 1) |

Dimensions & Location of terminal block

E2-1P2/1P5/101/2P2/2P5/201:

Unit: mm

DEMESIONS MODELABCDEFG E2-1P2/1P5/101/2P2/2P5/2011321161308.21186172

NOTE: For safety reason, we strongly recommend

users to remove the M4 grounding screw,

then screw the enclosed “metal frame

grounding terminal” on the same location to

make a grounding bar to ensure good earth

protection.

See NOTE

E2-202/203/401/402/403:

See NOTE of Page 18

Dimensions & Installation of class B Filter

Inverter with class B filter mounted.

Inverter with class B filter  & Din rail mounted kit.

### Mounting Instructions

Din Rail Mounting Diagram

Step1-

Aim and insert the 4 retention ribs of the DIN Rail at the 4 holes in rear panel of inverter

Step2-

Push the DIN Rail forward until the middle rib grips firmly with back panel

Step1-

Use a small screwdriver inserting it into the middle rib of DIN Rail and press the screwdriver in order to remove the DIN Rail from inverter

Additional DIN Rail Installation

A mounting clamp and a 35mm width rail must be used to install the Drive on the rail.

Install Drive

Dismounting Drive

First place the groove on the back of module on the upper edge of din rail, and then push the module down to lock up position. Finally press the mounting plate upward into module.

1   Pull the mounting plate downward.

2   Rotate the inverter module to dismount it.

E2-1P2/1P5/101/2P2/2P5/201- N4X (IP65)TYPE:

102.41

188.96

204.82

173.38

116.57

UNIT : mm

117.23

133.57

133.00

E2-202/203/401/402/403-N4X(IP65)TYPE:

UNIT:mm/inch

198.90 mm

E2-1P2/1P5/101/2P2/2P5/201- - N4X(IP65)TYPE INSTALLATION :

NOTE :

1. POWER SWITCH , REV-0-FWD SWITCH AND

Potentiometer are only for E2-2P2~201- N4S TYPE

2. Power supply cable : #14 AGE (2.0m    )

3. Motor cable : #16 AGE (1.25m    )

4. Torque value of Screw :

(1). Power/Motor cable (plug in) Therminal : 5kg-cm(4.34 in-lb)

(2). Remote control wire : 4kg-cm(3.47 in-lb)

(3). Outer Cover (M4) : 6kg-cm(5.20 in-lb)

123.40 mm

㎡

㎡

CIRCUIT DIAGRAM

NOTE:

(1). Input source : single-phase(L1,L2, )ensuring

that it is connected to a 200/240V supply.

(2). Output Moter : three-phase(,T1,T2,T3).

(3). POWER SWITCH , REV-0-FWD SWITCH AND

Potentiometer are only for E2-2P2~201- N4S TYPE.

Caution :

‧Do not start or stop the inverter using the

main circuit power.

‧FOR E2-2P2~201- -N4S TYPE :

Please always remain REV-0-FWD switch at 0

position.    In order to keep inverter has no

running signal before power-on again after

power supply interrupted.Otherwise, injury may

result.

‧FOR E2-2P2~201- -N4S TYPE :

Please always remain RE or FW switch at OFF

position.    In order to keep inverter has no

running signal before power-on again after

power supply interrupted.Otherwise, injury may

result.

POWER

SWITCH

AC

200~240

50/60HZ

3

PHASE

IM

L1

L2

T1

T2

T3

FW

RE

12V

REV-0-FWD

SWITCH

10V

VI

0V

Potentiometer

E2-2P2/2P5/201- -N4 (WITHOUT SWITCH TYPE ) CONNECTIONS & EMC MOUNTING:

E2-2P2/2P5/201- -N4S (WITH SWITCH TYPE ) EMC MOUNTING & CONNECTIONS :

E2-202/203/401/402/403—N4X(IP65)TYPE INSTALLATION:

2.Power supply cable :         4. Torque value of  Screw :

E2-200 #12AWG(3.5mm²)       (1).Power/Motor cable(TM1,

E2-400 #16AWG(1.25mm²)          TM3)

3. Motor cable :                              Terminal : 8 kgf-cm(6.94 in-lb)

E2-200 #14AWG(2.0mm²)       (2).Remote control wire :

E2-400 #16AWG(1.25mm²)          4 kgf-cm(3.47 in-lb)

(3).Outer Cover (M4) :

8kgf-cm(6.94 in-lb)

NOTE:

1.POWER SWITCH,REV-0-FWD SWITCH AND Potentiometer are only for E2-202~403-N4S TYPE

CIRCUIT DIAGRAM

NOTE:

(1).Input source:single-phase(L1,L2,) ensuring

that it is connected to a 200/240 supply.

or three-plase(L1,L2,L3, )ensuring that it is

connected to a 200/240,380/460V supply.

(2).Output Motor:three-phase(,T1,T2,T3).

Caution:

．Do not start or stop the inverter using the main circuit power.

．FOR E2-202~403--N4S TYPE:

Please always remain REV-0-FWD switch at 0 position. In order to keep inverter has no running signal before power-on again after power supply interrupted.Otherwise, injury may result.

．FOR E2-202~403--N4 TYPE:

Please always remain RE or FW switch at OFF position. In order to keep inverter has no running signal before power-on again after power supply interrupted.Otherwise, injury may result.

.

E2-202/203/401/402/403—N4S(WITH SWITCH TYPE)CONNECTIONS&EMC MOUNTING:

CONNECTION

EMC MOUNTING

E2-202/203/401/402/403—N4 (WITHOUT SWITCH TYPE)CONNECTIONS&EMC MOUNTING:

CONNECTION

EMC MOUNTING

Chapter 3 Software Index

# Keypad operating instructions

## Keypad Description

▲

▼

RESET

DATAENT

RUNSTOP

DSPFUN

POWER LED

CAUTION

▲ ▲ ▼ Do not operate keypad by screwdriver or other sharp-ended tool to avoid damaging keypad. (FREQ) * 1 (FREQ) DSP. FUN. DATA. ENT.

▲

▼

Brief keypad operation flowchart

F ××

DSP

FUN

F ××

×××

(READ)

END

×××

DATA

ENT

AFTER

0.5 SEC

▲

▼ ▼▼

* 2

(WRITE)

Note 1: Displayed setting of frequency when stopped. Display output frequency when running.

Note 2: The setting of the frequency can be modified either when stopped or when running.